EENG 5560 HW 2 Report Template

Name: Olasubomi Borishade

Due: 02/13/2026

Table of Contents

Design	3

Block diagrams	3

Overall design	3

Subcomponents	4

Design explanation	6

Structure and data flow	6

Functionality	6

Design Choices	6

Results	7

Generated Schematics	7

Top Level Design Results	7

Waveforms	7

Table/Calculations	8

Subcomponent Results	9

Waveforms	9

Table/Calculations	9

References	10

# Design

## Block diagrams

### ARCA Logic Array Overall Design

Figure 1 – ARCA Logic Array with overall ports, subcomponents and intermediate signals shown and heterogeneous interconnect

Overall component: ARCA Logic Array

Parameters:  	d_w – data width (for inputs and outputs)

rows – number of rows (of computation unit array)

cols – number of columns (of computation unit array)

Input ports:

| Port name | Dimensions/Bit width | Purpose |
| --- | --- | --- |
| A, B | 1 x 4 / 3-bit | Data inputs |
| OpSelA | 2 x 2 / custom type | Operation select input for arithmetic logic blocks |
| OpSelR, OpSelC | 1 x 2 / custom types | Operation select input for comparison and rotational logic blocks |
| ADataFlow1, ADataFlow2 | 1 x 2 / 2-bit | Input selector array for each A and B input of the second row of arithmetic logic blocks |
| CDataFlow1, CDataFlow2 | 1 / 2-bit | Input selector array for the A and B input of the comparison logic block in the second row |
| RDataFlow1, RDataFlow2 | 1 / 2-bit | Input selector array for the A and B input of the rotational logic block in the second row |

Output ports:

| Port name | Dimensions/Bit width | Purpose |
| --- | --- | --- |
| Y | 1 x 4 / 3-bit | Data output |

Necessary intermediate signals:

| As, Bs | 1 x 4 / 3-bit | A and B inputs of each computation unit in the second row |
| --- | --- | --- |
| Ys | 1 x 4 / 3-bit | Outputs of each computation unit |

Subcomponents:

| # of instances | Component type |
| --- | --- |
| 4 | Arithmetic Logic Block |
| 2 | Comparison Logic Block |
| 2 | Rotational Logic Block |

### Subcomponents

Figure 2 – Arithmetic Logic Block

Subcomponent: Arithmetic Logic Block

Input ports:

| Port name | Dimensions/Bit width | Purpose |
| --- | --- | --- |
| A, B | 1 x 1 / 3-bit | Data inputs |
| OpSelA | 1 x 1 / custom type | Operation select input |

Output ports:

| Port name | Dimensions/Bit width | Purpose |
| --- | --- | --- |
| Y | 1 x 1 / 3-bit | Data output |

Subcomponent: Comparison Logic Block

Input ports:

| Port name | Dimensions/Bit width | Purpose |
| --- | --- | --- |
| A, B | 1 x 1 / 3-bit | Data inputs |
| OpSelC | 1 x 1 / custom type | Operation select input |

Output ports:

| Port name | Dimensions/Bit width | Purpose |
| --- | --- | --- |
| Y | 1 x 1 / 3-bit | Data output |

Subcomponent: Rotational Logic Block

Input ports:

| Port name | Dimensions/Bit width | Purpose |
| --- | --- | --- |
| A, B | 1 x 1 / 3-bit | Data inputs |
| OpSelR | 1 x 1 / custom type | Operation select input |

Output ports:

| Port name | Dimensions/Bit width | Purpose |
| --- | --- | --- |
| Y | 1 x 1 / 3-bit | Data output |

## Design explanation

### Structure and data flow

The ARCA logic array consists of 3 different logic blocks arranged in a 2-by-4 matrix with heterogenous interconnects. The outer columns of the matrix are the arithmetic logic blocks, while the inner two are rotational and combination logic blocks. These blocks each have two 4-bit inputs, A and B, and a 4-bit output, Y. Each unit also has an operation selector, OpSel’X’, for the type of operation executed on the inputs.

The second row of the array is connected heterogeneously with the first row. The inner columns comprising rotational and comparison blocks are connected using 4:1 multiplexers, capable of taking an input from either of the logic blocks in the first row. The outer two columns are only able to take inputs from the logic block directly above it as well as its nearest neighbor using 2:1 multiplexers. This gives us a total of 8 multiplexers. The inputs, ADataFlow1 and ADataFlow2, RDataFlow1 and RDataFlow2, CDataFlow1 and CDataFlow2 are the array of select line input values deciding which output of the first row is received by the inputs of the second row. ‘X’DataFlow1 controls the A inputs and ‘X’DataFlow2 the B inputs. After the second row is computed, the result is then passed to the Y output of each logic block.

### Functionality

EACH ARITHMETIC LOGIC BLOCK’S INPUT A & B

| Opsel (Operation Select Input) | Y (Resulting Output) |
| --- | --- |
| aADD | A + B |
| aSUB | A – B |
| aMULT | A * B |
| NoOp | 000 |

EACH COMPARISON LOGIC BLOCK’S INPUT A & B

| Opsel (Operation Select Input) | Y (Resulting Output) |
| --- | --- |
| cGTH | If A > B, Y = 111, else 000 |
| cLTH | If A < B, Y = 111, else 000 |
| cEQT | If A = B, Y = 111, else 000 |
| cGTET | If A > B or A = B, Y = 111, else 000 |
| cLTET | If A < B or A = B, Y = 111, else 000 |
| cNET | If A /= B, Y = 111, else 000 |
| NoOp | 000 |

EACH ROTATIONAL LOGIC BLOCK’S INPUT A & B

| Opsel (Operation Select Input) | Y (Resulting Output) |
| --- | --- |
| rLSL | Shift A to the left by B number of bits |
| rLSR | Shift A to the right by B number of bits |
| NoOp | 000 |

SELECT BITS FOR THE ARITHMETIC LOGIC BLOCKS 2ND ROW ‘A’ INPUTS

| ADataFlow1 (Select Inputs) | ‘A’ inputs on the first and last columns of the array from the Y outputs on the first row |
| --- | --- |
| 00 | As (0) = Y (0, 0) & As (3) = Y (0, 2) |
| 01 | As (0) = Y (0, 0) & As (3) = Y (0, 3) |
| 10 | As (0) = Y (0, 1) & As (3) = Y (0, 2) |
| 11 | As (0) = Y (0, 1) & As (3) = Y (0, 3) |
| Other | As (0) = '000’ & As (3) = ‘000’ |

SELECT BITS FOR THE ARITHMETIC LOGIC BLOCKS 2ND ROW ‘B’ INPUTS

| ADataFlow2 (Select Inputs) | ‘B’ inputs on the first and last columns of the array from the Y outputs on the first row |
| --- | --- |
| 00 | Bs (0) = Y (0, 0) & Bs (3) = Y (0, 2) |
| 01 | Bs (0) = Y (0, 0) & Bs (3) = Y (0, 3) |
| 10 | Bs (0) = Y (0, 1) & Bs (3) = Y (0, 2) |
| 11 | Bs (0) = Y (0, 1) & Bs (3) = Y (0, 3) |
| Other | Bs (0) = '000’ & Bs (3) = ‘000’ |

SELECT BITS FOR THE ROTATIONAL LOGIC BLOCKS 2ND ROW ‘A’ INPUTS

| RDataFlow1 (Select Inputs) | ‘A’ inputs for the rotational logic block on the second row from the Y outputs on the first row |
| --- | --- |
| 00 | As (1) = Y (0, 0) |
| 01 | As (1) = Y (0, 1) |
| 10 | As (1) = Y (0, 2) |
| 11 | As (1) = Y (0, 3) |
| Other | As (1) = '000' |

SELECT BITS FOR THE ROTATIONAL LOGIC BLOCKS 2ND ROW ‘B’ INPUTS

| RDataFlow2 (Select Inputs) | ‘B’ inputs for the rotational logic block on the second row from the Y outputs on the first row |
| --- | --- |
| 00 | Bs (1) = Y (0, 0) |
| 01 | Bs (1) = Y (0, 1) |
| 10 | Bs (1) = Y (0, 2) |
| 11 | Bs (1) = Y (0, 3) |
| Other | Bs (1) = '000' |

SELECT BITS FOR THE COMPARISON LOGIC BLOCKS 2ND ROW ‘A’ INPUTS

| CDataFlow1 (Select Inputs) | A inputs for the comparison logic block on the second row from the Y outputs on the first row |
| --- | --- |
| 00 | As (2) = Y (0, 0) |
| 01 | As (2) = Y (0, 1) |
| 10 | As (2) = Y (0, 2) |
| 11 | As (2) = Y (0, 3) |
| Other | As (2) = '000' |

SELECT BITS FOR THE COMPARISON LOGIC BLOCKS 2ND ROW ‘B’ INPUTS

| CDataFlow2 (Select Inputs) | B inputs for the comparison logic block on the second row from the Y outputs on the first row |
| --- | --- |
| 00 | Bs (2) = Y (0, 0) |
| 01 | Bs (2) = Y (0, 1) |
| 10 | Bs (2) = Y (0, 2) |
| 11 | Bs (2) = Y (0, 3) |
| Other | Bs (2) = '000' |

As seen by the table, these logic blocks each have two 4-bit inputs, A and B, and a 4-bit output, Y. Each unit has an operation selector, Opsel’X’, for the type of operation executed on the inputs (e.g., aADD performs the arithmetic addition of inputs A and B).

The two inputs, DataFlow1 and DataFlow2, for each logic block type are the select line input values deciding which output of the first row is received by the inputs of the second row.

### Design Choices

The 8 multiplexers are also included in the top-level design through case statements and are not instantiated as subcomponents. There are no storage units in this design. Nothing is left as a high impedance output. The dataflow routing seems different than homework 1 for the arithmetic logic blocks, but it is the same concept for the other two logic block types.

# Results

## Generated Schematics

Figure 4 - ARCA Logic Array schematic

The ARCA logic array consists of 3 different logic blocks arranged in a 2-by-4 matrix with heterogenous interconnects. The outer columns of the matrix are the arithmetic logic blocks, while the inner two are rotational and combination logic blocks. These blocks each have two 4-bit inputs, A and B, and a 4-bit output, Y. Each unit also has an operation selector, OpSel’X’, for the type of operation executed on the inputs. The computation units are shown in blue.

The second row of the array is connected heterogeneously with the first row. The inner columns comprising rotational and comparison blocks are connected using 4:1 multiplexers, capable of taking an input from either of the logic blocks in the first row. The outer two columns are only able to take inputs from the logic block directly above it as well as its nearest neighbor using 2:1 multiplexers. This gives us a total of 8 multiplexers. The inputs, ADataFlow1 and ADataFlow2, RDataFlow1 and RDataFlow2, CDataFlow1 and CDataFlow2 are the array of select line input values deciding which output of the first row is received by the inputs of the second row. ‘X’DataFlow1 controls the A inputs and ‘X’DataFlow2 the B inputs. After the second row is computed, the result is then passed to the Y output of each logic block. The multiplexers are the yellow trapezoidal frames.

Figure 5 - Arithmetic Logic Block schematic

Figure 6 - Rotational Logic Block schematic

Figure 7 - Comparison Logic Block schematic

## Top Level Design Results

### Waveforms

Figure 8 – Top level Waveforms for Test Case 1

Figure 9 – Top level Waveforms for Test Case 2

### Table/Calculations

Test Case 1 (0ns to 900s):

| LB (0, 0) | 011 | 010 | aMULT | 110 | 110 | Yes |
| --- | --- | --- | --- | --- | --- | --- |
| LB (0, 1) | 110 | 011 | rLSR | 011 | 011 | Yes |
| LB (0, 2) | 101 | 111 | cLTH | 111 | 111 | Yes |
| LB (0, 3) | 011 | 011 | aADD | 110 | 110 | Yes |
| LB (1, 0) | 110 | 011 | aSUB | 011 | 011 | Yes |
| LB (1, 1) | 111 | 011 | rLSL | 000 | 000 | Yes |
| LB (1, 2) | 110 | 110 | cGTET | 111 | 111 | Yes |
| LB (1, 3) | 111 | 110 | aSUB | 001 | 001 | Yes |

Test Case 2 (0ns to 900s):

| LB (0, 0) | 001 | 011 | aADD | 100 | 100 | Yes |
| --- | --- | --- | --- | --- | --- | --- |
| LB (0, 1) | 001 | 001 | rLSL | 010 | 010 | Yes |
| LB (0, 2) | 100 | 011 | cNET | 111 | 111 | Yes |
| LB (0, 3) | 001 | 101 | aMULT | 101 | 101 | Yes |
| LB (1, 0) | 010 | 100 | aADD | 110 | 110 | Yes |
| LB (1, 1) | 101 | 010 | rLSR | 001 | 001 | Yes |
| LB (1, 2) | 111 | 010 | cGTH | 111 | 111 | Yes |
| LB (1, 3) | 111 | 101 | aSUB | 010 | 010 | Yes |

As can be seen by the table, the waveform output values for both test cases match the calculated values. This means the design works properly.

## Subcomponent Results

### Arithmetic Logic Block Waveforms

Figure 10 – Arithmetic Logic Block Test Case 1

Figure 11 – Arithmetic Logic Block Test Case 2

### Table/Calculations

Test Case 1 (0ns to 16ns):

| A | B | Oper | Calculated Op | Simulated Op | Match |
| --- | --- | --- | --- | --- | --- |
| 011 | 010 | aADD | 101 | 101 | Yes |
| 011 | 010 | aSUB | 001 | 001 | Yes |
| 011 | 010 | aMULT | 110 | 110 | Yes |
| 011 | 010 | NoOp | 000 | 000 | Yes |

T est Case 2 (0ns to 16ns):

| A | B | Oper | Calculated Op | Simulated Op | Match |
| --- | --- | --- | --- | --- | --- |
| 110 | 000 | aADD | 111 | 111 | Yes |
| 110 | 000 | aSUB | 111 | 111 | Yes |
| 110 | 000 | aMULT | 000 | 000 | Yes |
| 110 | 000 | NoOp | 000 | 000 | Yes |

### Rotational Logic Block Waveforms

Figure 12 – Rotational Logic Block Test Case 1

Figure 13 – Rotational Logic Block Test Case 2

### Table/Calculations

Test Case 1 (0ns to 16ns):

| A | B | Oper | Calculated Op | Simulated Op | Match |
| --- | --- | --- | --- | --- | --- |
| 001 | 010 | rLSL | 100 | 100 | Yes |
| 001 | 010 | rLSR | 000 | 000 | Yes |
| 001 | 010 | NoOp | 000 | 000 | Yes |

T est Case 2 (0ns to 16ns):

| A | B | Oper | Calculated Op | Simulated Op | Match |
| --- | --- | --- | --- | --- | --- |
| 010 | 001 | rLSL | 100 | 100 | Yes |
| 010 | 001 | rLSR | 001 | 001 | Yes |
| 010 | 001 | NoOp | 000 | 000 | Yes |

### Comparison Logic Block Waveforms

Figure 14 – Comparison Logic Block Test Case 1

Figure 15 – Comparison Logic Block Test Case 2

### Table/Calculations

Test Case 1 (0ns to 16ns):

| A | B | Oper | Calculated Op | Simulated Op | Match |
| --- | --- | --- | --- | --- | --- |
| 011 | 010 | cGTH | 111 | 111 | Yes |
| 011 | 010 | cLTH | 000 | 000 | Yes |
| 011 | 010 | cEQT | 000 | 000 | Yes |
| 011 | 010 | cGTET | 111 | 111 | Yes |
| 011 | 010 | cLTET | 000 | 000 | Yes |
| 011 | 010 | cNET | 111 | 111 | Yes |
| 011 | 010 | NoOp | 000 | 000 | Yes |

T est Case 2 (0ns to 16ns):

| A | B | Oper | Calculated Op | Simulated Op | Match |
| --- | --- | --- | --- | --- | --- |
| 101 | 101 | cGTH | 000 | 000 | Yes |
| 101 | 101 | cLTH | 000 | 000 | Yes |
| 101 | 101 | cEQT | 111 | 111 | Yes |
| 101 | 101 | cGTET | 111 | 111 | Yes |
| 101 | 101 | cLTET | 111 | 111 | Yes |
| 101 | 101 | cNET | 000 | 000 | Yes |
| 101 | 101 | NoOp | 000 | 000 | Yes |

# References

Hauck, S., & DeHon, A. (2008). Reconfigurable computing: The Theory and Practice of FPGA-based Computation. Morgan Kaufmann Pub.