POWER ELECTRONICS

FINAL PROJECT PROPOSAL

By:

Robert Valois & Olasubomi Borishade

Department of Electrical Engineering

University of North Texas, Denton

Discovery Park

EENG 4580.001 - Power Electronics

Dr. King Man Siu

September 30th, 2024

Table of Contents

Introduction	3

Project Selection & Contemporary Solutions	4

Target Topology & Specification	5

References	6

# Introduction

In our coursework, we’ve covered the basics of power electronics and the different types of topologies used to supply power to electronic systems. Simply put, power electronics is the science behind the processing/control of the flow of electric energy in a system to provide the system load with suitable voltages and currents.

A basic power electronics system consists of the power source or input, an input filter, the power converter or “processor,” the control system for the processor, and the output filter/output. Depending on the conversion process taking place in this system, different names are given to the circuitry (converter) used to achieve the target load specification:

- AC-DC conversion process, which is called the rectifier

- DC-DC conversion, which is called the chopper

- DC-AC conversion, which is called the inverter

- AC-AC conversion, which is called the cyclo converter/matrix converter

These four converters can be designed using different power semiconductors and passive electronic components. Thus far, we’ve only covered the first three conversion processes, studying the circuit analysis and overall system state per the different topologies used to achieve the conversion. For example, we began with the DC-DC conversion, which consists of many topologies (e.g., buck, boost, buck-boost, SEPIC, Zeta, Çuk, etc.).

All the lectures and assignments have given us the knowledge to begin conceptualizing our final project for the semester. We’ve been given three different options – each one is one of the three conversion methods we’ve studied:

- Multi-terminal DC-DCs (24Vdc PV input, 24Vdc Bi-directional battery, and 36Vdc output)

- AC-DC LED Driver (120Vac input, 16 LEDs, 48Vdc output, with dimming)

- PV inverter (0-200Vdc panels, 120Vac output)

Our goal is to focus on what would present us with a decent challenge while gaining the necessary experience required in power electronics. Dealing with both alternating and direct currents gives us a good balance. With one of the group members’ experiences in the energy efficiency industry within the lighting field, our options were thus narrowed down as only one would help build upon previous knowledge and place a demand on the group's innovation.

# Project Selection & Contemporary Solutions

For the final project in this class, of all the options, we have decided to go with the AC-DC LED Driver, which takes a 120Vac input (most likely a line input from a regular wall outlet) and converts it to a controllable (hence, dimmable) 48Vdc output to power 16 LEDs. This system should ensure that the LEDs function with stability and support the efficient conversion of energy to increase the lifespan of the LEDs. This type of circuit depends mainly on the topology and, thus, the control system utilized.

Modern applications of power electronics to power LEDs include:

- AC reverse buck LED driver [1]

- AC reverse buck-boost LED driver [1]

- Inverted buck converter-based LED driver [2]

- Offline flyback converter LED driver [3]

- Active clamp flyback converter LED driver [4]

The reverse-buck LED driver is supplied through a bridge rectifier, which technically makes its input a DC input. This DC input drives the power semiconductor used for switching. The input current is used for feedback to give a closer phase alignment to the power supply and ensure a higher power factor [1]. Reverse Buck would be the most straightforward implementation. Still, there may be a significant part of the AC cycle where the input is not driving the output: the voltage at the output of the rectifier at some point in each cycle is 0V, and this topology only functions when the power supplied is significantly more than its power generated at the output.

The AC reverse buck-boost is an intelligent method that solves the issue of the reverse buck topology, as it allows the input voltage to be below or above the output voltage. Because of its functionality, it provides a negative voltage output [1], but that doesn’t pose much of an issue in the grand scheme of things because polarity placement can be reversed.

The inverted buck converter-based LED configuration consists of two inverted buck converters with an EMI filter, an H-bridge rectifier, and ICs for control [2]. The use of two-buck converters reduces the average voltage. However, this approach would require a more extensive circuit, as more components are needed.

The offline flyback [3] and active clamp [4] converters provide sound isolation from the input using a transformer. However, compared with the aforementioned topologies, these occupy a decent amount of space on the board.

# Target Topology & Specification

Therefore, we’ve decided that the AC reverse buck-boost gives us the best, most balanced approach to supply power optimally to the LEDs.

Input Specifications:

- Input Voltage: 120Vac ± 10% (typical line input from a wall outlet)

- Input Frequency: 60 Hz (North American standard)

- Power Factor: ≥ 0.85 (to meet energy efficiency standards)

- Efficiency: ≥ 80% (under typical load conditions)

Output Specifications:

- Output Voltage: 48Vdc ± 5%

- Output Power: 24W max (assuming LEDs are driven at 1W each)

- Output Current: 0.5A (max)

- Ripple Voltage: ≤ 5% peak-to-peak (to ensure smooth LED operation and dimming performance)

- Ripple Current: ± 5% (for precise control of LED brightness and protection)

Dimming Control:

- Dimming Range: 0% to 100%

- Dimming Method: PWM (Pulse Width Modulation)

- PWM Frequency: 500Hz to 1kHz

- Dimming Response Time: < 100 ms (for smooth transitions during dimming adjustments)

- Control Input: Potentiometer or simple analog signal (0V – 5V)

# References

- Pellegrini, L. (2020, July 28). Popular converter topologies of LED drivers - technical articles. EEPower. https://eepower.com/technical-articles/popular-converter-topologies-of-led-drivers/#

- Vasudevamurt, S., & Ranjitha, R. (2018, July). A Modified Inverted Buck Converter based AC-DC LED Driver for Lighting Applications. https://www.jetir.org/papers/JETIR180Z002.pdf

- Saliva, A. A. (2013, January). Design Guide for Off-line Fixed Frequency DCM Flyback Converter. Mouser. https://www.mouser.com/pdfdocs/2-8.pdf

- Understanding and Designing an Active Clamp Current Mode Controlled Converter Using the UCC2897A. TI. (2020, January). https://www.ti.com.cn/cn/lit/an/slua535a/slua535a.pdf?ts=1724847319678